Article

A rapid, sensitive, low-cost assay for detecting hydrogenotrophic methanogens in anaerobic digesters using loop-mediated isothermal amplification

Anna M. Alessi1*, Bing Tao2, Wei Zhang2, Yue Zhang2, Sonia Heaven2, Charles J. Banks2 and James P. J. Chong1#

- Department of Biology, University of York, Wentworth Way, York, YO10 5DD, UK

- Water and Environmental Engineering Group, Faculty of Engineering and Physical Sciences, University of Southampton, University Road, Southampton, SO17 1BJ, UK

*	Current address: Biorenewables Development Centre Ltd, 1 Hassacarr Close, Chessingham Park, Dunnington, York, YO19 5SN, UK

# Correspondence should be addressed to: Professor James Chong Email: james.chong@york.ac.uk Tel: +44 (0)1904 328628

Abstract: Understanding how the presence, absence and abundance of different microbial genera supply specific metabolic functions for anaerobic digestion (AD) and how these impact on gas production is critical for a long-term understanding and optimization of the AD process. The strictly anaerobic methanogenic archaea are essential for methane production within AD microbial communities.  Methanogens are a phylogenetically diverse group that can be classified into three metabolically distinct lineages based on the substrates they use to produce methane. While process optimization based on physicochemical parameters is well established in AD, measurements that could allow manipulation of the underlying microbial community are seldom used as they tend to be non-specific, expensive, or time-consuming, or a combination of all three. Loop-mediated Isothermal Amplification (LAMP) assays combine a simple, rapid, low-cost detection technique with high sensitivity and specificity. Here we describe the optimization of LAMP assays for the detection of four different genera of hydrogenotrophic methanogens: Methanoculleus, Methanothermobacter, Methanococcus and Methanobrevibacter spp. By targeting archaeal elongation factor 2 (aEF2) these LAMP assays provide a rapid, low cost presence/absence indication of hydrogenotrophic methanogens that could be used as a real-time measure of process conditions. The assays were shown to be sensitive to 1 pg of DNA from most tested methanogen species, providing a route to a quantitative measure through simple serial dilution of samples. The LAMP assays described here offer a simple, fast and affordable method for the specific detection of four different genera of hydrogenotrophic methanogens. Our results indicate that this approach could be developed into a quantitative measure which could provide rapid, low-cost insight into the functioning and optimization of AD and related systems.

Keywords: hydrogenotrophic methanogens; anaerobic digestion; loop-mediated isothermal amplification; metagenomes

1. Introduction:

In addition to its vital and well-established role in stabilization of wastewater biosolids, anaerobic digestion (AD) is now increasingly used for resource recovery from a wide variety of agri- and food-processing wastes and industrial effluents. Key to this is the anaerobic microbial community that facilitates the degradation of organic materials through hydrolysis of large biopolymers.  Hydrolysis of polymers leads, via a cascade of metabolic pathways, to the production of small molecules and ultimately to carbon dioxide and methane, the main energy-carrying component of biogas and, a nutrient-rich digestate residue (Kougias and Angelidaki 2018). This process is mediated by hundreds of different interacting microbial species. The presence, absence and abundance of different genera affects the specific metabolic functions or “services” available to the overall microbial community.  Understanding how these functions impact on the interlinked processes of solids degradation and gas production offers new opportunities for optimization and enhancement of conventional AD. AD processes will also be an essential element in future biorefineries, and greater understanding of these community synergies and interdependencies may lead to new technologies based on exploiting the metabolic versatility of anaerobic consortia for generation of bio-based building block products.

One critical component of the microbial community in AD for methane production is the strictly anaerobic methanogenic archaea. Methanogens are a phylogenetically diverse group that can be classified into three metabolically distinct lineages (hydrogenotrophic, acetoclastic and methylotrophic, (Thauer et al. 2008)) based on the substrates used to produce methane. Hydrogenotrophic methanogens (Eq. 1) reduce CO2 to CH4 using H2. The acetoclastic methanogenesis pathway is responsible for two-thirds of biogenic methane production, but has been reported only for organisms within the order Methanosarcinales (Eq. 2) while methylotrophs (Eq. 3) can generate CH4 from a variety of chemical compounds containing methyl group such as methanol, methylsulfides or methylamines.

| Eq. 1 | 4 H2 + CO2 → CH4 + 2 H2O | ΔGo′ = –131 kJ mol−1 |
| --- | --- | --- |
| Eq. 2 | CH3COOH → CH4 + CO2 | ΔGo′ = –36 kJ mol−1 |
| Eq. 3 | CH3OH + H2 → CH4 + H2O | ΔGo′ = –112.5 kJ mole-1 |

In typical wastewater biosolids digesters the acetoclastic and hydrogenotrophic pathways dominate methane production and are usually associated with particular genera. For example, all members of the order Methanosarcinales (including the genera Methanosarcina, Methanosaeta and Methanolobus) contain genes for c-type cytochrome and methanophenazine (a functional menaquinone analogue) which enable them to utilize a broad range of substrates including hydrogen, formate and acetate. In contrast, hydrogenotrophic methanogens of the orders Methanopyrales, Methanococcales (e.g. genus Methanococcus), Methanobacteriales (e.g. genera Methanothermobacter, Methanobrevibacter) and Methanomicrobiales (e.g. genus Methanoculleus) do not contain a cytochrome system and are limited to growing mainly on H2 and CO2, although many can also grow on formate (Thauer et al. 2008; Galand et al. 2005). Hydrogen-scavenging methanogens can partner with syntrophic acetate-oxidizing bacteria (SAO) to support stable methane production in AD under certain conditions (Westerholm, Moestedt, and Schnürer 2016). The presence and relative abundance of hydrogenotrophs is of particular importance for the reductive biomethanisation of CO2 from both inorganic and organic sources, e.g. as a means of in situ biogas upgrading (Tao et al. 2019) or for ex situ carbon capture and utilization (Angelidaki et al. 2018). Recent studies reported microbial community shifts from Methanoculleus and Methanoregulaceae species towards hydrogenotrophic methanogenesis when ex-situ configurations were employed under thermophilic and mesophilic conditions, respectively (Bassani et al. 2017). Similarly, Kougias et al. (Kougias, Treu, et al. 2017) observed hydrogen-assisted methanogenesis with Methanothermobacter thermautotrophicus as the dominant member of the archaeal community in ex-situ reactors. Other studies showed that hydrogenotrophs are more tolerant to high ammonia concentrations than acetoclastic methanogens and hence more abundant in digesters treating high-ammonia feedstocks (Müller et al. 2016).

While physicochemical monitoring is well established as a basis for process optimisation in AD, measurements indicating the presence / absence of particular microbial community members, or that quantify these and could facilitate manipulation of the underlying microbial community tend to be non-specific, expensive and time-consuming, or a combination of all three. Culturing methods for anaerobic species are relatively inexpensive but require sophisticated infrastructure and are time consuming as they rely on the growth rates of the organisms being assessed. This is compounded by our inability to grow many species in isolation (Lagier et al. 2016).  Molecular methods such as PCR-based techniques (including qPCR) are sensitive but require expensive equipment and reagents and take a number of hours. Approaches such as 16S rRNA amplicon or whole metagenome high-throughput sequencing provide excellent specificity, but are also expensive, slow (days to weeks) and require a high level of expertise for data analysis and interpretation.

Loop-mediated Isothermal Amplification (LAMP) assays combine a simple, rapid, low-cost detection technique with high sensitivity and specificity.  LAMP assays are an efficient diagnostic tool, can be easily performed as they require a minimal amount of equipment and utilize low-cost reagents.  LAMP assays are widely employed in the food industry and in clinical settings where they have been used to detect specific pathogens including bacteria, parasites, fungi and viruses (Notomi et al. 2000; Wong et al. 2018).  LAMP relies on identifying a conserved gene within a targeted group of microorganisms against which a series of four to six primers must be designed (Table 1, cartoon). The assay is performed, often in less than one hour, by amplifying the target gene using Bst polymerase under isothermal conditions. A range of detection methodologies have been reported including visual identification of turbidity, colorimetric or fluorescent DNA interchelators, and end-point agarose gel electrophoresis (Wong et al. 2018).

This work aimed to determine the feasibility of using LAMP assays for rapid generation of diagnostic data on the status of a methanogenic microbial community with the notion that this information could be used to inform and improve decisions regarding process control in AD and related processes. We identified a candidate target gene to test the ability of LAMP to provide discriminating information on hydrogenotrophic methanogens. The suitability of this gene for our application was evaluated in silico and confirmed in vitro by PCR amplification of the external primer pair before being deployed in LAMP assays. We compared the results we gained from other methanogen-containing samples such as freshwater sediment, human stool samples and soil to samples from AD systems to further demonstrate the discriminatory power of our assays.

2. Materials and methods:

2.1. Identification of LAMP assay biomarkers

An unpublished AD metagenomics dataset (BTR1 A-H and BTR2 A-H, part of PRJEB27206 project submitted to the European Nucleotide Archive with sample accession numbers ERR2642234 – ERR2642249) sequenced using the Illumina HiSeq platform was used to identify candidate target genes for the LAMP assay. Briefly, reads were assembled with MEGAHIT (Li et al. 2015), ORFs (n=230,339) were predicted using PROKKA (Seemann 2014) and assigned KEGG ontology and taxonomy using GhostKoala (Kanehisa, Sato, and Morishima 2016)(Kanehisa, Sato, and Morishima 2016). KEGG orthologues identified uniquely as archaeal (n=434) were further examined by UniProt (Bateman et al. 2017) to identify proteins belonging to different hydrogenotrophic methanogen orders (Methanomicrobiales, Methanobacteriales and Methanococcales). These were aligned using UniProt and identity between proteins was noted (April 2020). Candidate gene nucleotide sequences from different methanogenic genera (Methanoculleus sp., Methanothermobacter sp., Methanococcus sp., Methanobrevibacter sp.) were retrieved from the NCBI database. Predicted ORF homologues from our unpublished assembly were also included in the analysis. Nucleotide sequences were aligned using CLC Genomics Workbench v.8.5.1. Highly conserved regions of genes were identified by visual inspection of the alignment and selected based on a threshold of >80% identity per nucleotide position to establish nucleotide consensus sequences (Supplementary Figure 1).

2.2. LAMP primer design

Consensus sequences of conserved regions of target genes were uploaded to Primer Explorer V5 (Kimura et al. 2011) and LAMP primers were design using default setting (length: F1c/B1c = 20 – 22 bp, F2/B2 = 18 – 20 bp, F3/B3 = 18 – 20 bp, Tm: F1c/B1c = 64 – 66, F2/B2 = 59 – 61, F3/B3 = 59-61, GC rate (%) = 40 – 65, dG threshold: 5’ stability = -3, 3’ stability= -4, dimer check - -2.5, distances: F2-B2 = 120 – 180, Loop(F1c-F2) 40 – 60, F2-F3 = 0 – 20, F1c-B1c = 0 – 100, Table 1). Loop primers were designed by the Primer Explorer V5 or manually by visual evaluation of the archaeal elongation factor 2 (aEF-2) genes alignments (Supplementary Figure 1).  Primer sequences were searched against the NCBI database to confirm their specificity for selected hydrogenotrophic genera. Desalted primers were synthesised by IDT Oligos. Primers were re-suspended in sterile nuclease-free water upon arrival and stored in 50 μl aliquots at -20°C.

Table 1. LAMP primer sequences targeting various hydrogenotrophic methanogen species.

| Target genus | Oligo name* | Oligo sequence (5' to 3')# | Tm [°C] | Length [bp] |
| --- | --- | --- | --- | --- |
| Methanoculleus (Mcu)a | F3_aEF2_Mcu | TAYCTBATCAACATGATYGAT | 48.0 | 21 |
|  | FLc_aEF2_Mcu | ACCGCRTCCACSACRAC | 58.4 | 17 |
|  | FIP_aEF2_Mcu | GTCTCCGTCTGKGGCATGGTTTTTGACGTGACYCGYGCCATG | 70.9 | 42 |
|  | B3_aEF2_Mcu | TYYTCGTTCATDCCCTTGAT | 52.3 | 20 |
|  | BL_aEF2_Mcu | ACGAGCAGGAGATGCAGATC | 56.8 | 20 |
|  | BIP_aEF2_Mcu | ACCGGCTGRTCAACGAGTTTTTTGACCTTRTCGATCAC | 65.8 | 38 |
| Methanothermobacter (Mth)b | F3_aEF2_Mth | ATTAAGGAGCTCATGTACCA | 50.6 | 20 |
|  | FLc_aEF2_Mth | CAAGGAARCGCTGATCCC | 54.9 | 18 |
|  | FIP_aEF2_Mth | CCTTGCCTGTTCCTGTTCTTTTGACAACCTCCTGGCTGGTGC | 69.7 | 42 |
|  | B3_aEF2_Mth | GCATTATRCCCTCAACKGC | 54.0 | 19 |
|  | BL_aEF2_Mth | TCAATGGTCCACTCCTA | 49.1 | 17 |
|  | BIP_aEF2_Mth | ACCATTGACGCTGCRAACGTTTTTCCTCATKGCCCTTGTAACGT | 69.1 | 44 |
| Methanococcus (Mco)c | F3_aEF2_Mco | ATGGGAAGAAGAGCAAAAATG | 51.3 | 21 |
|  | FLc_aEF2_Mco | ATCATWCCWGCWCCTGC | 52.0 | 17 |
|  | FIP_aEF2_Mco | CAAGYTGGTCTCCWGCTTTTCATYGACCACGGTAAAAC | 65.1 | 38 |
|  | B3_aEF2_Mco | CTCATTGCTCTTGTAACGTC | 51.1 | 20 |
|  | BL_aEF2_Mco | CTGCAAACGTKTCAATGGT | 52.7 | 19 |
|  | BIP_aEF2_Mco | GAAGAAGCWGCAAGAGGTATTTTTTCAACGTGACCWGGGGTRTC | 66.4 | 44 |
| Methanobrevibacter (Mbr)d | F3_aEF2_Mbr | GAAACTGTAYTYAGACAA | 43.5 | 18 |
|  | FLc_aEF2_Mbr | TCAGGAGCCATGTTYTTGATTA | 53.1 | 22 |
|  | FIP_aEF2_Mbr | GCTGAACCRAAWGCTACACTTTTTAATCAACGARTTAAAATTA | 60.6 | 43 |
|  | B3_aEF2_Mbr | AAGTGTTCTACTACCATAC | 45.6 | 19 |
|  | BL_aEF2_Mbr | ATYATTGATTAYTGTAATG | 39.4 | 19 |
|  | BIP_aEF2_Mbr | TGGGCTATYAAYGTTCCTTTTGGTACTTTTTKAGCTAATTC | 61.7 | 41 |

a - CP000562.1: Methanoculleus marisnigri JR1, NC_018227.1 (HE964772.2): Methanoculleus bourgensis MS2T, LT158599.1: Methanoculleus sp. MAB1, LT549891.1: Methanoculleus bourgensis isolate BA1. b - CP001710.1 (NC_014408.1): Methanothermobacter marburgensis str. Marburg, AP011952.1: Methanothermobacter sp. CaT2, AE000666.1 (NC_000916.1): Methanothermobacter thermautotrophicus str. Delta H, LT608329.1: Methanothermobacter wolfeii isolate SIV6, c - CP026606.1: Methanococcus maripaludis strain DSM 2067, CP002913.1: Methanococcus maripaludis X1, CP000609.1: Methanococcus maripaludis C5, CP000745.1: Methanococcus maripaludis C7, CP000867.1: Methanococcus maripaludis C6, CP002057.1: Methanococcus voltae A3, d - CP000678.1: Methanobrevibacter smithii ATCC 35061, CP004050.1: Methanobrevibacter sp. AbM4, CP010834.1: Methanobrevibacter sp. YE315. * F3 – forward outer primer, FL- forward loop primer, FIP – forward inner primer which consists of the F2 region (at the 3' end) complementary to the F2c region, and the same sequence as the F1c region at the 5' end, B3 – backward outer primer, BL – backward loop primer, BIP – backward inner primer consists of the B2 region (at the 3' end) complementary to the B2c region, and the same sequence as the B1c region at the 5' end. # - F(B)1c primer is in italics, linker is in bold and F(B)2 primer is underlined in F(B)IP primers

2.3. Template DNA

Bacterial cultures of Escherichia coli ATCC11775, Staphylococcus epidermidis ATCC14990, and Bacillus sp. 3PL were grown in 5 ml nutrient broth cultures overnight at 37°C with shaking. Genomic DNA was extracted using the DNeasy UltraClean Microbial kit (Qiagen #12224). Methanoculleus marisnigri JG1 were grown under anaerobic condition in the medium 141 as described previously (Romesser et al. 1979).  Methanococcus maripaludis S1 was cultured in McCas liquid media as described previously (Haydock et al. 2004).  Methanothermobacter thermautotrophicus (DSMZ 1053) cultures were grown under chemoautotrophic conditions (Nölling, Frijlink, and de Vos 1991), with H2 and CO2 as the sole energy and carbon sources. Genomic DNA from Methanoculleus marisnigri JG1, Methanothermobacter thermautotrophicus and Methanococcus maripaludis S1 was extracted from 10 ml cultures using a DNeasy UltraClean Microbial kit (Qiagen #12224) with initial bead beating increased to 20 minutes.  Environmental DNA (eDNA) was extracted from 200 mg of biomass using a DNeasy PowerSoil kit (Qiagen #12888) from samples collected from a 1686 m3 anaerobic digester operated by a local water company (NAB), a lab-scale (1L) mesophilic anaerobic digester with hydrogen addition for in situ CO2 biomethanisation (BTR, University of Southampton), a lab-scale thermophilic anaerobic digester fed on food waste (EX, University of Southampton) and garden topsoil and salt marsh sediment samples (F30 and S30). Stool samples (P) were collected from human volunteers with approval from the Biology Ethics Committee (University of York).  Metagenomic DNA was extracted from 250 mg stool using the Qiagen QIAamp PowerFecal DNA kit (#12830). The concentration of genomic and metagenomic DNA was measured using a Nanodrop-3000. Template samples were diluted to 10 ng/μl with sterile, nuclease-free water.  Samples were stored at -20°C.

2.4. PCR with outer F3/B3 primers

25 μl reactions consisting of 5 μl 5x reaction buffer, 0.5 μl 10 mM dNTP, 1.25 μl 10 μM each forward and reverse primers, 0.25 μl Phusion polymerase (2000 U/ml, NEB, #M0530S), 2 μl template (10 ng/μl) and sterile nuclease-free water (NEB) were assembled on ice. Initial denaturation was performed at 98°C for 30 sec, followed by 25 cycles of denaturation at 98°C for 10 sec, annealing at various temperatures for 10 sec and extension at 72°C for 15 sec. A final extension was performed at 72°C for 5 minutes.  Products were examined by 1.5% (w/v) agarose gel electrophoresis in 1x TAE separated at 110 V for 45 minutes.  PCR amplification products were purified (ZymoResearch DNA Clean and Concentrator-25 kit, #D4005) and sequenced using the F3 (10 μM) outer primer using the GATC Eurofins Supreme Run service. Sequencing products were analysed using BLASTn (Altschul et al. 1990) (Supplementary Table 1).

2.5. LAMP assay

LAMP reactions were prepared in single 0.2 ml PCR tubes in a laminar-flow hood. Each experiment was repeated independently three times.  A “10x primer mix” of LAMP primers consisting of 2 μM F3 and B3 primers, 16 μM FIP and BIP primers and 4 μM FL and BL primers was prepared, aliquoted and stored at -20°C.  Bst 3.0 reactions consisted of: 2.5 μl 10x reaction buffer, 1.5 μl 100 mM MgSO4, 3.5 μl 10 mM dNTPs, 2.5 μl 10x primer mix, 1 μl Bst 3.0 enzyme (NEB, #M0374S), 2 μl 10 ng/µl DNA.  Bst 2.0 Warm Start reactions consisted of: 12.5 μl 2x Warm Start master mix (NEB #E1700S), 2.5 μl 10x primer mix, 2 μl 10 ng/µl DNA.  All reactions were assembled on ice. A no-template control was included to ensure amplification specificity. Reactions were incubated at either 65°C for 30 minutes (Mth, Mco), 65°C for 45 minutes (Mcu) or 58°C for 45 minutes (Mbr) before the enzyme was heat inactivated at 80°C for 5 minutes. Product detection was performed by carefully transferring 9 μl of the stopped reaction to a fresh 0.2 ml PCR tube and adding 1 μl of 1000 x SYBR Green I dye (ThermoFisher Scientific). Visual inspection was performed with positive reactions turning yellow/green while negative reactions remained orange. DNA template amplification was additionally visualised by loading 5 μl of product on a 1.5% (w/v) agarose gel in 1x TAE following electrophoresis at 110 V for 45 minutes.

2.6. LAMP sensitivity

10 ng/µl of genomic DNA was diluted by 10-fold serial dilution using sterile nuclease-free water to a concentration of 0.001 ng/µl. LAMP reactions with Mcu and Mco primers were performed in technical duplicates using 2.5 μl 10x reaction buffer, 1.5 μl 100 mM MgSO4, 3.5 μl 10 mM dNTPs (source), 2.5 μl 10x primer mix (see LAMP assay section), 1 μl Bst 3.0 enzyme (NEB, #M0374S) and 1 μl (10 ng/μl – 0.001 ng/μl) DNA.  Bst 2.0 Warm Start reactions for Mth consisted of: 12.5 μl 2x Warm Start master mix (NEB #E1700S), 2.5 μl 10x primer mix, 2 μl 10 ng/µl DNA.  Reactions were incubated at either 65°C for 30 minutes (Mth, Mco) and 65°C for 45 minutes (Mcu) before the enzyme was heat inactivated at 80°C for 5 minutes. End-point products were visualised as described in the LAMP assay section.

3. Results

3.1. aEF-2 is a potential biomarker for hydrogenotrophic methanogen detection in LAMP assays

To identify hydrogenotrophic methanogen sequences, we sampled and sequenced a lab scale anaerobic digester receiving a supplementary feed of hydrogen to promote in situ CO2 biomethanisation  (Tao et al. 2019). The resulting metagenomic dataset of 225,144,076 reads was assembled using the pipeline described above into 136,759 contigs from which we identified 230,339 putative open reading frames. We identified 434 KEGG orthologues uniquely present in Archaea from the ORFs. These orthologues included assignments to cellular processes (n=6), environmental information processing (n=24), genetic information processing (n=139) and metabolism (n=132).

Archaeal elongation factor 2 (aEF-2, aka fusA, K03234) was identified as a good target gene for the LAMP assay based on a number of highly conserved regions across the gene that facilitated primer design. In particular, a high level of protein sequence identity (> 40%) was observed within different methanogen orders. A Uniprot alignment of the 24 available Methanobacteriales aEF-2 protein sequences showed 47% identity which included 356 identical positions. Similarly, an alignment of the 17 available aEF-2 Methanococcales sequences showed 61% identity with 447 identical positions and Methanomicrobiales (n=9) aEF-2 proteins shared 61% identity with 449 identical positions. We included novel putative aEF-2 sequences in our analyses that we identified from our metagenomic assembly as related to the genus Methanoculleus (Supplementary Figure 1, designated as CODx_xxxx). We confirmed that the aEF-2 gene was a good LAMP assay candidate by comparing these results to similar searches that we performed for other potential gene targets including genes involved in methanogenesis pathways: methyl-CoA reductase (mcrA), methylene tetrahydromethanopterin reductase (mer), energy-converting hydrogenase A (ehaA), the 30S ribosomal protein S28e (rps28e) and 50S ribosomal protein L44e (rpl44e)  (Table 2).

Table 2. Analysis of potential gene targets for the LAMP assay for detection of hydrogenotrophic methanogens in environmental samples.

|  | ID [%] | # sequences (Uniprot) | # identical positions | # similar positions |
| --- | --- | --- | --- | --- |
| aEF-2 (K03234) |  |  |  |  |
| Methanomicrobiales | 61 | 9 | 449 | 157 |
| Methanobacteriales | 47 | 24 | 356 | 192 |
| Methanococcales | 61 | 17 | 447 | 172 |
| mcrA (K00399) |  |  |  |  |
| Methanomicrobiales | 57 | 15 | 325 | 132 |
| Methanobacteriales | 45 | 41 | 250 | 157 |
| Methanococcales | 66 | 22 | 366 | 117 |
| mer (K00320) |  |  |  |  |
| Methanomicrobiales | 63 | 9 | 212 | 70 |
| Methanobacteriales | 34 | 24 | 110 | 105 |
| Methanococcales | 58 | 17 | 192 | 75 |
| ehaA (K14097) |  |  |  |  |
| Methanomicrobiales | 36 | 6 | 63 | 38 |
| Methanobacteriales | 23 | 21 | 48 | 51 |
| Methanococcales | 36 | 17 | 62 | 40 |
| rsp28e (K02979) |  |  |  |  |
| Methanomicrobiales | 75 | 9 | 52 | 10 |
| Methanobacteriales | 63 | 24 | 46 | 16 |
| Methanococcales | 57 | 17 | 44 | 22 |
| rpl44e (K02929) |  |  |  |  |
| Methanomicrobiales | 61 | 9 | 56 | 18 |
| Methanobacteriales | 53 | 24 | 50 | 24 |
| Methanococcales | 51 | 17 | 49 | 15 |

3.2. Genus-specific PCR products are generated with LAMP outer primers

To demonstrate that the outer LAMP primers were unique and specific to the aEF-2 genes in targeted methanogenic species, the outer forward (F3) and reverse (B3) primers were used in a standard PCR reaction. Three pairs of primers that were designated as the outer primers for future LAMP assays designed to target selected Methanoculleus, Methanothermobacter and Methanococcus species (primers F3/B3_Mcu, F3/B3_Mth, F3/B3_Mco) were shown to correctly amplify aEF-2 genes when provided with gDNA extracted from single species cultures (Figure 1 a-c). Sequencing of these products confirmed that only aEF-2 genes were amplified (Table S1). No amplification of bacterial negative controls (Escherichia coli ATCC11775, Staphylococcus epidermidis ATCC14990, and Bacillus sp. 3PL) was observed with primers that targeted Methanococcus and Methanoculleus species (Figure 1a and 1b). Primers targeting Methanothermobacter species aEF-2 showed non-specific amplification with template DNA from Escherichia coli and Methanoculleus marisnigri (Figure 1c, lane 5). Several weak bands were also noted in the eukaryotic negative control performed with F3/B3_aEF2_Mth primers (Figure 1c, lane 7). In the absence of a pure culture of Methanobrevibacter, eDNA extracted from eight human stool samples was used as a template as this organism is expected in about one in three of the population. Amplification was observed for two samples indicating that the F3/B3 Mbr primers target an aEF-2 gene (Figure 1d). The products generated by pair F3/B3 primers, were sequenced and confirmed correct affiliation with selected methanogenic species (Supplementary Table S1).

Figure 1. Agarose gel electrophoresis analysis of PCR products of reactions performed with F3 and B3 outer primers and selected bacterial, archaeal and fungal DNA as templates.  All the components in the PCR reactions were the same except for F3/B3 primers which were (a) Methanoculleus (Mcu), (b) Methanothermobacter (Mth), (c) Methanococcus (Mco), (d) Methanobrevibacter (Mbr). Bacterial DNA templates (blue line) from Escherichia coli (lane 1), Staphylococcus epidermidis (lane 2), and Bacillus sp. 3PL (lane 3). Archaeal DNA templates (orange line) from Methanococcus maripaludis (lane 4), Methanoculleus marisnigri (lane 5) and Methanothermobacter thermoautotrophicus (lane 6), eukaryotic DNA was extracted from wheat root rhizobiome (green line, lane 7). For (d) eDNA from stool samples was used as a template (yellow line, lanes 1-8). A non-template control (NTC) was included in all assays.

3.3. Specificity of full LAMP assays

The same template DNAs used to test the outer primers in PCR were used to further examine the specificity of our designed primers in full LAMP assays. LAMP primer sets correctly amplified aEF-2 genes from specific methanogen genomic DNA producing a characteristic ladder-like pattern. Bacterial and eukaryotic DNA control reactions showed no amplification, indicating that the primers were specific for selected species of methanogens (Figure 2).

Figure 2. LAMP assay outputs comparing agarose gel electrophoresis (top of panels) and Sybr Green I end-point detection (bottom of panels).  Specific F3/B3 primers and genomic DNA from (a) Methanoculleus, (b) Methanothermobacter, (c) Methanococcus, (d) Methanobrevibacter were used in assays performed with either Bst 3.0 (a, b, d) or Bst 2.0 (c).  Bacterial DNA (blue line) originated from Escherichia coli (lane 1), Staphylococcus epidermidis (lane 2), and Bacillus sp. 3PL (lane 3), archaeal DNA (orange line) originated from Methanococcus maripaludis (lane 4), Methanoculleus marisnigri (lane 5) and Methanothermobacter thermoautotrophicus (lane 6), eukaryotic DNA was extracted from wheat root rhizobiome (green line, lane 7). ). For (d) eDNA extracted from two Methanobrevibacter positive (see Figure 1) stool samples was used as a template (yellow line). A non-template control (NTC) was included in all assays.

3.4. A set of LAMP assays to discriminate between hydrogenotrophic methanogenic genera

We searched our genus-targeted LAMP primer sets against the NCBI database using BLAST adjusted for short sequences. In a few cases, specifically where sequences made use of a high level of degenerate nucleotides, we found that individual primers could potentially anneal to aEF-2 genes from more than one genus. We also noted that some primers could potentially anneal to elongation factor 2 genes from various eukaryotic species. These were discounted as insignificant, as our earlier results (Figures 1, 2) demonstrated no amplification of eukaryotic EF-2 genes with either outer primer pairs or whole primer sets. Overall, our in silico evaluation showed that the majority of our primers theoretically annealed to aEF-2 genes only from the targeted genera (Figure 3).

Figure 3. Specificity of LAMP primer sets for targeted genera within the methanogenic archaea.  Evolutionary relationships of elongation factor 2 (aEF-2) genes within hydrogenotrophic methanogens were conducted in MEGA7 and inferred using Neighbour-Joining.  Evolutionary distances were computed using the Jukes-Cantor method as number of base substitutions per site.  The analysis used 59 nucleotide sequences.  All positions containing gaps and missing data were eliminated.  There were a total of 1971 positions in the final dataset.  An optimal tree with sum of branch length = 6.51376856 is shown.  The percentage of replicate trees in which the associated taxa clustered together in the bootstrap test (based on 500 replicates) are annotate branches.  The tree is drawn to scale, with branch lengths in the same units as the evolutionary distances used to infer the phylogenetic tree.  LAMP primer sets predicted to target specific species are shown as coloured circles (cyan: Methanococcus; blue: Methanoculleus, red: Methanothermobacter; green: Methanobrevibacter).

Complete LAMP reactions were used to screen a range of samples for the presence of hydrogenotrophic methanogenic genera using two different Bst formulations. We extracted DNA from samples of mesophilic and thermophilic anaerobic digesters and compared these to DNA extracted from salt marsh sediment, soil and human stool samples. A characteristic ladder-like pattern using Bst 3.0 indicated that Methanoculleus species were detected in mesophilic anaerobic digester samples (Figure 4a). Assays for detection of Methanothermobacter species were initially performed using Bst 3.0. This led to false-positive amplification in non-template controls.  Reagent contamination resulting in false positive signals has previously been identified as a potentially confounding factor in LAMP assays (Karthik et al. 2014). Even after replacing all reagents, the Bst 3.0 chemistry continued to provide inconclusive results when combined with the Methanothermobacter primers. In contrast, the Bst 2.0 Warm Start DNA polymerase showed a distinct ladder-like pattern for Methanothermobacter positive controls and reliably detected Methanothermobacter species in thermophilic anaerobic digester samples (Figure 4b). Methanococcus was not detected in any of the environmental samples we tested but was successfully amplified using gDNA isolated from Methanococcus marisnigri JG1 used as a positive control with both Bst 3.0 and Bst 2.0 Warm Start polymerases (Figure 4c). Reaction conditions for Methanobrevibacter required further optimization. As we did not have pure cultures available as a source of genomic DNA for this genus, environmental samples that showed positive amplification with the outer F3/B3 primers (Figure 1, Figure 2d) were used to optimise the LAMP assays for these species. A 55°C to 65°C temperature gradient yielded an optimal assay temperature of 58°C for Methanobrevibacter LAMP using Bst 3.0 polymerase due to a lower melting temperature for the F3 and B3 primers. A positive ladder-like pattern was observed using metagenomic DNA extracted from human stool samples (Figure 4d).

Figure 4. LAMP assay outputs comparing agarose gel electrophoresis (top of panels) and Sybr Green I end-point detection (bottom of panels).  Assays used primers targeted to species with the genera Methanoculleus (a), Methanothermobacter (b), Methanococcus (c) and Methanobrevibacter (d).  Assays were performed with Bst 3.0 (a, c, d) or Warm Start Bst 2.0 chemistry (b). Genomic DNA (20 ng) from Methanoculleus (a), Methanothermobacter (b) or Methanococcus (c) was used as a positive control (lane 1, red line).  LAMP assays were performed with metagenomic DNA from process scale mesophilic digesters (lanes 2, 3), lab-scale mesophilic digesters (lanes 4, 5) or lab-scale thermophilic anaerobic digesters (lanes 6, 7; green line), salt marsh sediment (lanes 8, 9; orange line), garden soil (lane 10; orange line) or human stool samples (lanes 11, 12; blue line). A non-template control (NTC) was included in all assays.

3.5. LAMP sensitivity

Genomic DNA from the methanogens Methanoculleus marisnigri JG1, Methanothermobacter thermautotrophicus and Methanococcus maripaludis was used to determine the potential sensitivity of our methanogen LAMP assays (Figure 5). Decreasing amounts of gDNA from M. marisnigri, M. thermautotrophicus and M. maripaludis cultures were used to evaluate the sensitivity of individual LAMP assays for detecting these hydrogenotrophic methanogen genera. Methanoculleus and Methanococcus detection was successful for samples ranging from 10 ng/ul (10 ng) of gDNA to 0.001 ng/ul (1 pg). The Methanothermobacter detection limit was 0.1 ng/ul. Methanobrevibacter primer sensitivity was not determined as gDNA for this genus was not available.

Figure 5. LAMP assay outputs comparing agarose gel electrophoresis (top panels) and Sybr Green I end-point detection (bottom panels).  Genomic DNA from (a) Methanoculleus marisnigri JG1 (Mcu), (b) Methanothermobacter thermautotrophicus (Mth) and (c) Methanococcus maripaludis S1 (Mco) were added in decreasing quantities (lane 1: 10ng, lane 2: 1 ng, lane 3: 100 pg, lane 4: 10 pg, lane 5: 1 pg) to genus-specific LAMP assays and compared to non-template controls (lane NTC) to determine assay sensitivity.

4. Discussion

In this study, we aimed to develop a low cost, simple, qualitative approach to the detection of selected hydrogenotrophic methanogens in samples from anaerobic digesters operating under a variety of conditions. These protocols could be equally applied to other samples likely to contain methanogens such as environmental microbiomes or high-rate reactors for ex situ CO2 biomethanisation. Our LAMP assays provide a rapid indication of the presence or absence of targeted methanogenic genera and do not rely on expensive equipment or extensive post-assay analysis as might be the case with more traditional sequence-based methods. We targeted genera with multiple cultured representatives and publicly available genome sequences to aid primer design. Target genes for LAMP assays require a high degree of conservation at the nucleotide level to facilitate primer design and should ideally be single copy genes that are unique to the microbial group of interest. Gao and Gupta (Gao and Gupta 2007) reported 31 proteins uniquely found in various methanogenic archaea. These proteins include well-characterized biomarkers such as enzymes encoded by the mcr (methyl coenzyme M reductase (Friedrich 2005)) and mtr (methyl-H4MPT coenzyme M methyltransferase (Enzmann et al. 2018)) genes involved in methanogenesis and several hypothetical proteins with domains of unknown function. A lack of sequence data in public repositories for the hypothetical genes precluded their use in LAMP assay design. By conducting comparative alignments of different proteins (Table 2) from selected genera of hydrogenotrophic methanogens, we identified archaeal elongation factor 2 (aEF-2), as a good candidate for LAMP. EF-2 has been used in several studies to evaluate the relationships between Archaea, Bacteria, and Eukaryotes (Iwabe et al. 1989; Elkins et al. 2008; Williams et al. 2012; Raymann, Brochier-Armanet, and Gribaldo 2015).  More recent studies showed EF-2 paralogs in various Archaea and Eukaryotes suggesting that EF-2 might not be an effective single copy marker gene (Narrowe et al. 2018) although archaeal EF-2 gene has been widely used as a phylogenetic marker for Archaea. We confirmed that LAMP primer sets reliably amplified archaeal EF-2 from selected genera of hydrogenotrophic methanogens. Although some of our primers aligned to eukaryotic EF-2 genes, which could lead to false-positive results for environmental samples containing highly abundant archaeal and eukaryotic species, no amplification was observed using environmental DNA extracted from a wheat root rhizobiome, suggesting that our designed primer sets are specific for the targeted archaeal EF2 genes.

In testing, our LAMP assay successfully detected Methanoculleus species in samples from process-scale wastewater biosolids digesters (Figure 4a, lanes 1,2) consistent with metagenomic sequencing results of the same samples  (Alessi et al., in preparation) and as expected from studies using different detection methods (Kirkegaard et al. 2017).  Methanoculleus species are common to bioreactors operating at high ammonia concentrations (Wang, Fotidis, and Angelidaki 2015), typically dominating anaerobic digestion communities in reactors processing manures (Campanaro et al. 2016). Consistent with other literature (Kougias, Treu, et al. 2017; Kougias, Campanaro, et al. 2017), we also detected Methanoculleus in lab-scale reactors supplemented with hydrogen during in situ CO2 biomethanisation (Figure 4a, lanes 4,5) but not in thermophilic food waste fed systems (Figure 4, lanes 6,7), indicating our primers are specific for mesophilic Methanoculleus species. Additionally, our LAMP assay results are consistent with 16S rRNA amplicon profiles obtained for these samples in our previous work. We showed that CO2 reductive biomethanisation systems were dominated by Methanoculleus genus hydrogenotrophs and that the abundance of these species increased with a rise in H2 addition to these systems (Tao et al. 2019).

We detected Methanothermobacter in lab-scale digesters treating food waste at 55°C using LAMP. Methanothermobacter species are generally found in thermophilic anaerobic digestate with growth temperatures ranging from 40-70 °C (Martin et al. 2013; Treu et al. 2018).

Our results show our assays are able to differentiate other hydrogenotrophic genera such as Methanococcus (Jones, Paynter, and Gupta 1983) and Methanobrevibacter. Although our positive control indicated assay specificity, we were unable to detect Methanococcus in the available salt marsh samples, possibly due to low microbial abundance resulting in DNA concentrations below the sensitivity of our LAMP assay. Consistent with 16S rRNA amplicon sequencing (Tao et al. 2019), Methanococcus was not detected by LAMP in mesophilic lab scale digesters (Figure 4c, lanes 4,5). Using DNA extracted from stool samples from human volunteers, we confirmed the presence of Methanobrevibacter by LAMP. Methanobrevibacter species are typically associated with the gastrointestinal tracts of herbivorous ruminants, termites and humans (Dridi et al. 2009; Leadbetter and Breznak 1996; Leahy et al. 2010). Thus our LAMP assays could potentially be used in a clinical setting as an alternative to a methane breath test.

The LAMP assays described here offer a simple, fast and affordable method for the specific detection of methanogens in anaerobic digestion and other samples. Given the different thresholds of sensitivity shown in Figure 5, our results indicate that with further work this approach could be developed into a rapid quantitative measure. For example, using quantified reference standards and serial dilutions, genera could be quantified to an order of magnitude in samples for less than 1 USD in an hour or less. Previous LAMP-based studies have focused on the detection of single species, whereas we have designed LAMP assays spanning three different orders (Methanobacteriales, Methanococcales and Methanomicrobiales) and four different hydrogenotrophic genera which should provide additional rapid, low-cost insight into the functioning of anaerobic digestion and related systems. Such insights in turn will provide a basis for guidance of process operations, and help to support the development of new areas of application from reductive CO2 biomethanisation to novel methane-based biorefineries.

Acknowledgments: This work was carried out with support from the Engineering and Physical Sciences Research Council through the IBCat H2AD project (EP/M028208/1).  JPJC is a Royal Society Industry Fellow (IF160022).  Thanks to Yorkshire Water for access to process-scale digester samples. Metagenomic DNA samples from human stool and salt marshes were kindly provided by Dr Farah Shahi and Fiona Llewellyn-Beard respectively.

Author Contributions: Conceptualization, Anna M. Alessi and James P.J. Chong; Data curation, Anna M. Alessi; Formal analysis, Anna M. Alessi; Funding acquisition, Sonia Heaven, Charles J. Banks and James P.J. Chong; Investigation, Anna M. Alessi; Methodology, Anna M. Alessi and James P.J. Chong; Project administration, Anna M. Alessi; Resources, Anna M. Alessi, Bing Tao, Wei Zhang, Yue Zhang, Sonia Heaven, Charles J. Banks and James P.J. Chong; Supervision, Sonia Heaven, Charles J. Banks and James P.J. Chong; Validation, Bing Tao, Wei Zhang, Yue Zhang, Sonia Heaven and Charles J. Banks; Visualization, Anna M. Alessi; Writing – original draft, Anna M. Alessi and James P.J. Chong; Writing – review & editing, Anna M. Alessi, Bing Tao, Wei Zhang, Yue Zhang, Sonia Heaven, Charles J. Banks and James P.J. Chong.

Conflicts of Interest: The sponsors had no role in the design, execution, interpretation, or writing of the study.

References

Angelidaki, Irini, Laura Treu, Panagiotis Tsapekos, Gang Luo, Stefano Campanaro, Henrik Wenzel, and Panagiotis G. Kougias. 2018. “Biogas Upgrading and Utilization: Current Status and Perspectives.” Biotechnology Advances 36 (2): 452–66. https://doi.org/10.1016/j.biotechadv.2018.01.011.

Bassani, Ilaria, Panagiotis G. Kougias, Laura Treu, Hugo Porté, Stefano Campanaro, and Irini Angelidaki. 2017. “Optimization of Hydrogen Dispersion in Thermophilic Up-Flow Reactors for Ex Situ Biogas Upgrading.” Bioresource Technology 234 (June): 310–19. https://doi.org/10.1016/j.biortech.2017.03.055.

Bateman, Alex, Maria Jesus Martin, Claire O’Donovan, Michele Magrane, Emanuele Alpi, Ricardo Antunes, Benoit Bely, et al. 2017. “UniProt: The Universal Protein Knowledgebase.” Nucleic Acids Research 45 (D1): D158–69. https://doi.org/10.1093/nar/gkw1099.

Campanaro, Stefano, Laura Treu, Panagiotis G. Kougias, Davide De Francisci, Giorgio Valle, and Irini Angelidaki. 2016. “Metagenomic Analysis and Functional Characterization of the Biogas Microbiome Using High Throughput Shotgun Sequencing and a Novel Binning Strategy.” Biotechnology for Biofuels 9: 26. https://doi.org/10.1186/s13068-016-0441-1.

Dridi, Bédis, Mireille Henry, Amel El Khéchine, Didier Raoult, and Michel Drancourt. 2009. “High Prevalence of Methanobrevibacter Smithii and Methanosphaera Stadtmanae Detected in the Human Gut Using an Improved DNA Detection Protocol.” PLOS ONE 4 (9): e7063. https://doi.org/10.1371/journal.pone.0007063.

Elkins, James G., Mircea Podar, David E. Graham, Kira S. Makarova, Yuri Wolf, Lennart Randau, Brian P. Hedlund, et al. 2008. “A Korarchaeal Genome Reveals Insights into the Evolution of the Archaea.” Proceedings of the National Academy of Sciences of the United States of America 105 (23): 8102–7. https://doi.org/10.1073/pnas.0801980105.

Enzmann, Franziska, Florian Mayer, Michael Rother, and Dirk Holtmann. 2018. “Methanogens: Biochemical Background and Biotechnological Applications.” AMB Express 8 (January). https://doi.org/10.1186/s13568-017-0531-x.

Friedrich, Michael W. 2005. “Methyl-Coenzyme M Reductase Genes: Unique Functional Markers for Methanogenic and Anaerobic Methane-Oxidizing Archaea.” Methods in Enzymology 397: 428–42. https://doi.org/10.1016/S0076-6879(05)97026-2.

Galand, P. E., H. Fritze, R. Conrad, and K. Yrjälä. 2005. “Pathways for Methanogenesis and Diversity of Methanogenic Archaea in Three Boreal Peatland Ecosystems.” Applied and Environmental Microbiology 71 (4): 2195–98. https://doi.org/10.1128/AEM.71.4.2195-2198.2005.

Gao, Beile, and Radhey S. Gupta. 2007. “Phylogenomic Analysis of Proteins That Are Distinctive of Archaea and Its Main Subgroups and the Origin of Methanogenesis.” BMC Genomics 8 (1): 86. https://doi.org/10.1186/1471-2164-8-86.

Haydock, Andrew K., Iris Porat, William B. Whitman, and John A. Leigh. 2004. “Continuous Culture of Methanococcus Maripaludis under Defined Nutrient Conditions.” FEMS Microbiology Letters 238 (1): 85–91. https://doi.org/10.1111/j.1574-6968.2004.tb09741.x.

Iwabe, N., K. Kuma, M. Hasegawa, S. Osawa, and T. Miyata. 1989. “Evolutionary Relationship of Archaebacteria, Eubacteria, and Eukaryotes Inferred from Phylogenetic Trees of Duplicated Genes.” Proceedings of the National Academy of Sciences of the United States of America 86 (23): 9355–59.

Jones, W. Jack, M. J. B. Paynter, and R. Gupta. 1983. “Characterization of Methanococcus Maripaludis Sp. Nov., a New Methanogen Isolated from Salt Marsh Sediment.” Archives of Microbiology 135 (2): 91–97. https://doi.org/10.1007/BF00408015.

Kanehisa, Minoru, Yoko Sato, and Kanae Morishima. 2016. “BlastKOALA and GhostKOALA: KEGG Tools for Functional Characterization of Genome and Metagenome Sequences.” Journal of Molecular Biology 428 (4): 726–31. https://doi.org/10.1016/j.jmb.2015.11.006.

Karthik, K., Rajesh Rathore, Prasad Thomas, T. R. Arun, K. N. Viswas, Kuldeep Dhama, and R. K. Agarwal. 2014. “New Closed Tube Loop Mediated Isothermal Amplification Assay for Prevention of Product Cross-Contamination.” MethodsX 1 (January): 137–43. https://doi.org/10.1016/j.mex.2014.08.009.

Kimura, Yasumasa, Michiel J. L. de Hoon, Shintaro Aoki, Yuri Ishizu, Yuki Kawai, Yasushi Kogo, Carsten O. Daub, Alexander Lezhava, Erik Arner, and Yoshihide Hayashizaki. 2011. “Optimization of Turn-Back Primers in Isothermal Amplification.” Nucleic Acids Research 39 (9): e59–e59. https://doi.org/10.1093/nar/gkr041.

Kirkegaard, Rasmus H., Simon J. McIlroy, Jannie M. Kristensen, Marta Nierychlo, Søren M. Karst, Morten S. Dueholm, Mads Albertsen, and Per H. Nielsen. 2017. “The Impact of Immigration on Microbial Community Composition in Full-Scale Anaerobic Digesters.” Scientific Reports 7 (1): 9343. https://doi.org/10.1038/s41598-017-09303-0.

Kougias, Panagiotis G., and Irini Angelidaki. 2018. “Biogas and Its Opportunities—A Review.” Frontiers of Environmental Science & Engineering 12 (3): 14. https://doi.org/10.1007/s11783-018-1037-8.

Kougias, Panagiotis G., Stefano Campanaro, Laura Treu, Xinyu Zhu, and Irini Angelidaki. 2017. “A Novel Archaeal Species Belonging to Methanoculleus Genus Identified via De-Novo Assembly and Metagenomic Binning Process in Biogas Reactors.” Anaerobe, Biogas Science 2016, 46 (Supplement C): 23–32. https://doi.org/10.1016/j.anaerobe.2017.02.009.

Kougias, Panagiotis G., Laura Treu, Daniela Peñailillo Benavente, Kanokwan Boe, Stefano Campanaro, and Irini Angelidaki. 2017. “Ex-Situ Biogas Upgrading and Enhancement in Different Reactor Systems.” Bioresource Technology 225 (February): 429–37. https://doi.org/10.1016/j.biortech.2016.11.124.

Lagier, Jean-Christophe, Saber Khelaifia, Maryam Tidjani Alou, Sokhna Ndongo, Niokhor Dione, Perrine Hugon, Aurelia Caputo, et al. 2016. “Culture of Previously Uncultured Members of the Human Gut Microbiota by Culturomics.” Nature Microbiology 1 (November): 16203. https://doi.org/10.1038/nmicrobiol.2016.203.

Leadbetter, J. R., and J. A. Breznak. 1996. “Physiological Ecology of Methanobrevibacter Cuticularis Sp. Nov. and Methanobrevibacter Curvatus Sp. Nov., Isolated from the Hindgut of the Termite Reticulitermes Flavipes.” Appl. Environ. Microbiol. 62 (10): 3620–31.

Leahy, Sinead C., William J. Kelly, Eric Altermann, Ron S. Ronimus, Carl J. Yeoman, Diana M. Pacheco, Dong Li, et al. 2010. “The Genome Sequence of the Rumen Methanogen Methanobrevibacter Ruminantium Reveals New Possibilities for Controlling Ruminant Methane Emissions.” PLOS ONE 5 (1): e8926. https://doi.org/10.1371/journal.pone.0008926.

Li, Dinghua, Chi-Man Liu, Ruibang Luo, Kunihiko Sadakane, and Tak-Wah Lam. 2015. “MEGAHIT: An Ultra-Fast Single-Node Solution for Large and Complex Metagenomics Assembly via Succinct de Bruijn Graph.” Bioinformatics (Oxford, England) 31 (10): 1674–76. https://doi.org/10.1093/bioinformatics/btv033.

Martin, Matthew R., Jeffrey J. Fornero, Rebecca Stark, Laurens Mets, and Largus T. Angenent. 2013. “A Single-Culture Bioprocess of Methanothermobacter Thermautotrophicus to Upgrade Digester Biogas by CO2-to-CH4 Conversion with H2.” Archaea 2013 (October): e157529. https://doi.org/10.1155/2013/157529.

Müller, Bettina, Li Sun, Maria Westerholm, and Anna Schnürer. 2016. “Bacterial Community Composition and Fhs Profiles of Low- and High-Ammonia Biogas Digesters Reveal Novel Syntrophic Acetate-Oxidising Bacteria.” Biotechnology for Biofuels 9: 48. https://doi.org/10.1186/s13068-016-0454-9.

Narrowe, Adrienne B., Anja Spang, Courtney W. Stairs, Eva F. Caceres, Brett J. Baker, Christopher S. Miller, and Thijs J. G. Ettema. 2018. “Complex Evolutionary History of Translation Elongation Factor 2 and Diphthamide Biosynthesis in Archaea and Parabasalids.” Genome Biology and Evolution 10 (9): 2380–93. https://doi.org/10.1093/gbe/evy154.

Nölling, Jörk, Maarten Frijlink, and Willem M. de Vos. 1991. “Isolation and Characterization of Plasmids from Different Strains of Methanobacterium Thermoformicicum.” Microbiology, 137 (8): 1981–86. https://doi.org/10.1099/00221287-137-8-1981.

Notomi, T., H. Okayama, H. Masubuchi, T. Yonekawa, K. Watanabe, N. Amino, and T. Hase. 2000. “Loop-Mediated Isothermal Amplification of DNA.” Nucleic Acids Research 28 (12): E63.

Raymann, Kasie, Céline Brochier-Armanet, and Simonetta Gribaldo. 2015. “The Two-Domain Tree of Life Is Linked to a New Root for the Archaea.” Proceedings of the National Academy of Sciences of the United States of America 112 (21): 6670–75. https://doi.org/10.1073/pnas.1420858112.

Romesser, J. A., R. S. Wolfe, F. Mayer, E. Spiess, and A. Walther-Mauruschat. 1979. “Methanogenium, a New Genus of Marine Methanogenic Bacteria, and Characterization OfMethanogenium Cariaci Sp. Nov. AndMethanogenium Marisnigri Sp. Nov.” Archives of Microbiology 121 (2): 147–53. https://doi.org/10.1007/BF00689979.

Seemann, Torsten. 2014. “Prokka: Rapid Prokaryotic Genome Annotation.” Bioinformatics (Oxford, England) 30 (14): 2068–69. https://doi.org/10.1093/bioinformatics/btu153.

Tao, Bing, Anna M. Alessi, Yue Zhang, James P. J. Chong, Sonia Heaven, and Charles J. Banks. 2019. “Simultaneous Biomethanisation of Endogenous and Imported CO2 in Organically Loaded Anaerobic Digesters.” Applied Energy 247 (August): 670–81. https://doi.org/10.1016/j.apenergy.2019.04.058.

Thauer, Rudolf K., Anne-Kristin Kaster, Henning Seedorf, Wolfgang Buckel, and Reiner Hedderich. 2008. “Methanogenic Archaea: Ecologically Relevant Differences in Energy Conservation.” Nature Reviews Microbiology 6 (8): 579–91. https://doi.org/10.1038/nrmicro1931.

Treu, L., P. G. Kougias, B. de Diego-Díaz, S. Campanaro, I. Bassani, J. Fernández-Rodríguez, and I. Angelidaki. 2018. “Two-Year Microbial Adaptation during Hydrogen-Mediated Biogas Upgrading Process in a Serial Reactor Configuration.” Bioresource Technology 264 (September): 140–47. https://doi.org/10.1016/j.biortech.2018.05.070.

Wang, Han, Ioannis A. Fotidis, and Irini Angelidaki. 2015. “Ammonia Effect on Hydrogenotrophic Methanogens and Syntrophic Acetate-Oxidizing Bacteria.” FEMS Microbiology Ecology 91 (11). https://doi.org/10.1093/femsec/fiv130.

Westerholm, Maria, Jan Moestedt, and Anna Schnürer. 2016. “Biogas Production through Syntrophic Acetate Oxidation and Deliberate Operating Strategies for Improved Digester Performance.” Applied Energy 179 (October): 124–35. https://doi.org/10.1016/j.apenergy.2016.06.061.

Williams, Tom A., Peter G. Foster, Tom M. W. Nye, Cymon J. Cox, and T. Martin Embley. 2012. “A Congruent Phylogenomic Signal Places Eukaryotes within the Archaea.” Proceedings. Biological Sciences 279 (1749): 4870–79. https://doi.org/10.1098/rspb.2012.1795.

Wong, Y.-P., S. Othman, Y.-L. Lau, S. Radu, and H.-Y. Chee. 2018. “Loop-Mediated Isothermal Amplification (LAMP): A Versatile Technique for Detection of Micro-Organisms.” Journal of Applied Microbiology 124 (3): 626–43. https://doi.org/10.1111/jam.13647.