# High concentrations of pharmaceuticals in a Nigerian river catchment

Olatayo M. Ogunbanwoa, Paul Kaya, Alistair B. Boxallb, John Wilkinsonb, Chris J. Sinclairc, Rasheed A. Shabid, Abolaji E. Fasasid , Gregory A. Lewisd , Olanrewaju A. Amodad and Lee E. Browna

a School of Geography/water@leeds, University of Leeds, Leeds, West Yorkshire, LS2 9JT, UK

bEnvironment Department, University of York, Heslington, York, North Yorkshire, YO10 5NG, UK

cEcoChemistry Section, FERA Science Ltd, Sand Hutton, York, North Yorkshire, YO41 1LZ, UK

dLagos State Environmental Protection Agency (LASEPA), Governor’s Road, Secretariat Alausa, Lagos State, Southwest Nigeria.

# *Author for correspondence: p.kay@leeds.ac.uk

Abstract

Pharmaceutical contamination of the environment is recognized as a global problem although work has focused on Europe and North America to date and there remains a dearth of information for developing countries, including those in Africa. To address this data gap the occurrence of thirty-seven pharmaceuticals belonging to nineteen therapeutic classes was monitored in surface water and effluent in Lagos State, Southwest Nigeria. Samples were collected quarterly from twenty-two sites and twenty-six compounds were detected at least once, many in the micrograms per litre range. Maximum concentrations for those compounds detected ranged from 75 to 129 µg L-1 and even mean concentrations for thirteen compounds were in the order of µg L-1. These values are amongst the highest ever measured globally.

Sewage effluent was more important than drug manufacturing waste in polluting rivers although there are likely to be numerous unregulated sources of effluent being discharged to rivers which require further study, including urban waste collection areas and vacuum trucks which collect effluent. Seasonal trends in the data were complex with some compounds being found at higher concentrations in the dry season and, conversely, others being greater during the wet period, this variation potentially relating to the variety of pollution sources in the catchment. Pharmaceuticals are indispensable to human health although their usage and discharge into the aquatic environment may lead to ecological problems and antibiotic resistance. The data presented in this paper indicate that pharmaceutical pollution of freshwaters is a serious issue in Nigeria and management efforts are needed to ameliorate this issue.

Keywords: Pharmaceuticals; emerging contaminants; rivers; water quality; pollution; sewage

1. Introduction

Pharmaceuticals were first detected in the environment in the 1970’s (Tabak and Bunch, 1970; Norpoth et al., 1973) and since then numerous studies have quantified their occurrence in aquatic systems. The majority of these studies have been undertaken in Europe and North America (Hughes et al., 2013) whilst far fewer have been performed in low to medium income countries, including those in Africa, South America and the Middle East (Hughes et al., 2013). A small number of studies have been undertaken in Africa and detection frequencies have been high at between 60 and 100 % (Ngumba et al., 2016) with concentrations typically greater than those measured in the West (Fekadu et al., 2019). In Kenya concentrations of up to 167 µg L-1 were found in sewage effluent and surface waters (K’Oreje et al., 2012, 2016, 2018) whilst in South Africa pharmaceuticals have also been found to be ubiquitous in effluent and freshwaters at concentrations ranging from ng L-1 to µg L-1 (Agunbiade and Moodley, 2015; Gumbi et al., 2017). Such high concentrations may be due to a number of factors including high drug usage and poor regulation of this, the presence of numerous pharmaceutical manufacturing plants, and poorly developed sewage treatment facilities (Fekadu et al., 2019). In Nigeria, analgesics, antibiotics, antacids, antihistamines, anticonvulsants, steroids, antimalarials and antihypertensives are among the most consumed classes of drugs and are routinely purchased without a prescription (Odunsanya, 2005). However, the statistics available on the usage of pharmaceuticals are not reliable because of the activities of unregistered pharmacies in some cities such as Lagos (Akande and Ologe, 2007; Oshikoya and Ojo, 2007; Nwolisa et al., 2006; Odunsanya, 2005). The current paper aims to add to the limited occurrence data for pharamceuticals in African effluent and surface water and, therefore, improve our understanding of the global importance of pharamceutical pollution. We report the results of a monitoring campaign to understand the occurrence of thirty-seven pharmaceuticals in the Odo-Iya Alaro River catchment in Lagos, Nigeria, a country where few pharmaceutical monitoring data are available. The main objectives were: (i) to understand the extent to which drugs belonging to different therapeutic classes are found in effluents and surface water, (ii) to quantify spatial and temporal patterns of pharmaceutical contamination and, (iii) to highlight particular compounds of environmental concern.

2. Methods

2.1. Substances monitored

The pharmaceuticals monitored (Table 1) were chosen based on their expected presence in surface waters (Burn et al., 2018), being high-use drugs that have been found previously in rivers around the world (Hughes et al., 2013).

2.2. Study catchment

The Odo-Iya Alaro River (Figure 1) forms a sub-catchment of the Ogudu river, which discharges into the Lagos lagoon, and flows through Ogba, Ikeja and Maryland which have a combined population of 2.5 million people. The catchment contains a sewage treatment plant (STP), three major pharmaceutical manufacturing plants and many smaller ones located in the industrial estates of Ogba and Ikeja which discharge their effluents through drainage pipes and canals into the river. Some of these canals pass through densely populated urban areas which discharge untreated domestic waste to them and raw sewage may also enter the river due to emptying of vacuum trucks which collect untreated effluent in urban areas (Ogunbanwo, 2011). Twenty-two sampling stations were located along the river based on accessibility and the potential to sample both receiving waters and effluents being discharged to them (Table S1).

| Table 1. Physico-chemical properties of the pharmaceutical compounds monitored in the Odo Iya Alaro river, Lagos, Nigeria (www.drugbank.ca) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Therapeutic Group | Compound | LogKow | pKa | Molecular Wgt (g ml-1) | Formula | Solubility (mg L-1) |
| Analgesic & Anti-inflammatory | Codeine | 1.19 | 8.21- 10.60 | 299.37 | C18H21NO3 | 9000 |
|  | Hydrocodone | 2.16 | 8.23 | 299.37 | C18H21NO3 | n/a |
|  | Paracetamol | 0.46-0.49 | 9.38 | 151.17 | C8H9NO2 | 14000 |
|  | Tramadol | 3.01 | 9.41 | 263.38 | C16H25NO2 | 630 |
| Antacid | Cimetidine | 0.40 | 6.80 | 252.34 | C10H16N6S | 9380 |
|  | Ranitidine | 0.27 | 8.08 | 314.40 | C13H22N4O3S | 24700 |
| Antiallergic | Loratadine | 5.20 | 5.00 | 382.89 | C22H23CIN2O2 | 0.011 |
| Antibiotics | Erythromycin | 3.06 | 8.88-8.90 | 733.94 | C37H67NO13 | 2000 |
|  | Sulfamethoxazole | 0.89 | 1.60-5.70 | 253.28 | C10H11N3O3S | 610 |
|  | Trimethoprim | 0.91 | 7.12 | 290.32 | C14H18N4O3 | 400 |
| Anticonvulsant | Carbamazepine | 2.45 | 13.90 | 236.27 | C15H12N2O | 17.7 |
|  | Gabapentin | -1.10 | 3.68-10.70 | 171.24 | C9H17NO2 | 4490 |
| Antidepressant | Amitriptyline | 4.92 | 9.40-9.76 | 277.41 | C20H23N | 9.71 |
|  | Desvenlafaxine | 2.72 | 10.11 | 263.38 | C16H25NO2 | 1400 |
|  | Diltiazem | 2.7 | 8.06 | 414.52 | C22H26N2O4S | 465 |
|  | Oxazepam | 2.24 | 10.90 | 286.72 | C15H11CIN2O2 | 179 |
|  | Venlafaxine | 3.20 | 10.09 | 277.41 | C17H27NO2 | 267 |
| Antihistamine | Diphenhydramine | 3.27 | 8.98 | 255.36 | C17H21NO | 3060 |
|  | Fexofenadine | 2.81 | 4.28-8.76 | 501.67 | C32H39NO4 | 0.024 |
|  | Ketotifen | 3.85 | 8.43 | 309.43 | C19H19NOS | 15.3 |
|  | Ceterizine | 1.70-3.57 | 3.58-7.74 | 388.89 | C21H25CIN2O3 | 65.8 |
| Antidiabetic | Metformin | -2.64 | 12.40 | 165.63 | C4H12CIN5 | n/a |
|  | Sitagliptin | 1.39 | 8.78 | 407.32 | C16H15F6N5O | 179.2 |
| Antipsychotic | Diazepam | 2.82 | 3.40 | 284.74 | C16H13CIN2O | 50 |
|  | Temazepam | n/a | -1.4-10.68 | 300.74 | C16H13CIN2O2 | 164 |
| Anti-malaria | Artemisinin | 2.90 | 4.60 | 282.22 | C15H22O5 | n/a |
| Antiarrhythmic | Lidocaine | 2.26 | 8.01 | 234.34 | C14H22N2O | 4100 |
| Antiretroviral | Lamivudine | -9.54 | -0.16-14.29 | 229.25 | C8H11N3O3S | 70000 |
| Antiviral | Oseltamivir | 0.95 | 7.70 | 312.41 | C16H28N2O4 | 1600 |
| Contraceptive | Norethisterone | 2.97 | -1.7-17.59 | 298.43 | C20H26O2 | 7.04 |
| Beta Blocker | Atenolol | 0.16 | 9.60 | 266.34 | C14H22N2O3 | 13300 |
|  | Propranolol | -0.45 | 9.42 | 259.35 | C16H21NO2 | 61.7 |
| SERM | Raloxifene | 6.09 | 7.99-9.92 | 473.59 | C28H27NO4S | 0.25 |
| Diuretics | Triamterene | 0.98 | 3.11-15.88 | 253.27 | C12H11N7 | 48.2 |
| Calcium-Channel Blocker | Verapamil | 3.83 | 8.92 | 454.61 | C27H38N2O4 | 4.47 |
| SSRIs | Sertraline | 4.30 | 9.47 | 306.23 | C17H17CI2N | 3.5 |
|  | Citalopram | 1.39 | 9.50 | 324.40 | C20H21FN20 | n/a |
| n/a = not available; Wgt =Weight |  |  |  |  |  |  |

Figure 1. The Odo-Iya Alaro river in Lagos State, Southwest Nigeria and the twenty-two sampling stations used in the current study.

2.3. Sample collection

Effluent and surface water samples were collected on a quarterly basis to incorporate both the wet (April and July) and dry seasons (October and January). Amber glass sampling vessels were rinsed with 100% methanol once and deionised water three times to remove potential contamination before sampling. Samples were collected at the same time of day and location, checked using a Global Positioning System (GPS). At each sampling site, three 50 mL water samples were collected and then homogenised into a single 150 mL composite sample. A 10 mL aliquot of each composite sample was then filtered on site through a Whatman GFF (0.7 µm pore size) glass microfiber syringe filter into a 20 mL amber glass vial with a Teflon-lined screw cap (Fisher Scientific, UK). Samples were frozen immediately with dry ice before shipping within 24 hrs to the UK for analysis. On arrival (three days), samples were thawed immediately and analysed.

2.4 Chemical analysis

Quantification of pharmaceutical concentrations was achieved using HPLC-MS/MS with a Thermo Scientific TSQ Endura Mass spectrometer coupled with an UltiMate 3000 liquid chromatograph. The method used was adapted from Furlong et al. (2014) and further validated (Burns et al., 2018). Briefly, prior to starting the quantitative analysis, 500 µL of each water sample was diluted with 495 µL of HPLC-grade water and spiked with 5 µL of a mixture of internal standards (each at a concentration of 80 µg L-1) in glass autosampler vials. The 50 % dilution was done in order to clean the samples and bring analyte concentrations to within the calibrated range. Where concentrations were found to still exceed the calibrated range further dilution and reanalysis was carried out. A random number generator was used to randomise the order in which samples were injected onto the HPLC-MS/MS. Analysis was conducted by direct injection of 100 µL of each sample into a Phenomenex Eclipse Plus C18 chromatography column using a Phenomenex C18 (ODS, Octadecyl) 4 mm x 3 mm ID guard column. Mobile phase A was HPLC-grade water with 0.01 M formic acid and 0.01 M ammonium formate while mobile phase B was 100 % HPLC-grade methanol. A flow rate of 0.45 mL min-1 was used with a gradient starting at 10 % B which then increased to 40 % at 5 min, 60 % at 10 min, 100 % at 15 min, and remaining 100 % B until 23 min then dropping to 10 % at 23 min prior to a re-equilibration. The autosampler temperature was kept at 4C and the HPLC column compartment at 40C. The collision gas was argon at a pressure of 2 mTorr. Quantification was done with a 16-point calibration using deuterated internal standards (Burns et al., 2018) ranging from 1 to 32000 ng L-1 and calibration r2-values were consistently >0.95. Analytical limits of detection were calculated as described by Burns et al. (2018) and ranged from 0.9 ng L -1 (carbamazepine) to 12.4 ng L-1 (gabapentin) (Table S2). Quality control (QC) measures were used throughout the analysis; briefly, method blanks (n=6) were made with an identical collection procedure as the environmental samples although using HPLC-grade water. Concentrations of the target pharmaceuticals were consistently below levels of analytical quantification in the method blanks. Additionally, QCs consisting of all target pharmaceuticals at a concentration of 80 ngL-1 were injected after every 4 samples followed by an instrumental blank consisting of pure HPLC-grade water. Analytical tolerance was consistently within ±15 % and the instrumental blanks did not contain detectable residues of the target analytes.

### 2.5 Data analysis

Data were organised using Excel (Microsoft, 2013) and residuals of the data were checked for normal distribution using the Shapiro-Wilk normality test and homogeneity of variance using the Bartlett test of homogeneity of variances. R (R Development Core Team, 2008) was used to analyse the data; general linear model and Chi-square were used to find if there were differences between the sampling sites. Seasonal variations were analysed using one-way ANOVA where assumptions of normality and homogeneity were met followed by Tukey’s post-hoc tests to determine if there is any variation in concentrations between the wet and the dry the seasons.

## 3. Results

### 3.1 Detection frequencies

All of the study compounds were detected although the frequency of detection varied greatly for different substances (Figure 2). Some, including carbamezapine, fexofenadine and paracetamol, were present in sewage effluent and surface waters most of the time whereas others, such as diltiazem, propranolol and venlafaxine, were rarely detected. Detection frequencies in pharmaceutical manufacturing effluent were significantly lower than in sewage effluent and river water (GLM: χ2 (3) = 883.32, p <0.001).

Figure 2. Detection frequencies for pharamceuticals measured in sewage effluent, pharmaceutical manufacturing waste and rivers (urban and semi-urban).

3.2 Pharmaceutical concentrations

Peak pharmaceutical concentrations were in the range of hundreds of micrograms per litre while mean concentrations were several orders of magnitude lower (Figure 3). The antibiotic sulfamethoxazole was detected at the highest concentration of 129.4 µg L-1 whilst paracetamol was also measured at 111.4 µg L-1. Paracetamol and sulfamethoxazole also had the highest mean concentrations of 18.2 and 11.2 µg L-1 respectively. Many other compounds were found at concentrations only slightly lower than these including cimetidine, fexofenadine, carbamazepine, metformin, diazepam, atenolol, trimethoprim, and codeine (Table S3).

Figure 3. Pharmaceutical concentrations measured in the Odo Iya Alaro river catchment, Lagos, Nigeria. Boxes represent median and 25th and 75th percentiles whilst whiskers show minimum and maximum values.

There was a significant difference in pharmaceutical concentrations in the different matrices sampled (GLM: χ2 (3) = 883.32, p <0.001). When pharmaceuticals were detected in manufacturing effluent concentrations tended to be higher than in sewage effluent and river water. Drugs were diluted slightly after sewage effluent had entered urban rivers although concentrations were lower still in semi-urban reaches. Although detection frequencies were generally higher in the wet season, concentrations were often higher in the dry season (Figure 4).

Figure 4. Comparison of pharmaceutical concentrations measured in the wet and dry seasons in different matrices in the Odo Iya Alaro river, Lagos, Nigeria. SEW = sewage effluent in wet season, SED = sewage effluent in dry season, PEW = pharmaceutical manufacturing effluent in wet season, PED = pharmaceutical manufacturing effluent in dry season, URW = urban river in wet season, URD = urban river in dry season, SURW = sub-urban river in wet season, SURD = sub-urban river in dry season. Boxes represent median and 25th and 75th percentiles whilst whiskers show minimum and maximum values.

## 4. Discussion

Pharmaceuticals are biologically active and pseudo-persistent in the environment due to the continual input of wastewater effluent to rivers (Hughes et al., 2013; Kay et al., 2017; Burns et al., 2018) and, therefore, potentially pose a toxicological risk to non-target organisms (Boxall et al., 2002; Huang et al., 2012). Monitoring has mainly taken place in Europe and the United States though and data are severely lacking for developing countries, such as those in Africa. This paper addresses this important research gap by presenting new information about the presence of pharmaceuticals in pharmaceutical manufacturing effluent, sewage effluent and surface water in Lagos, Nigeria, a country which has been studied very little.

The detection of twenty-six pharmaceuticals in the Odo Iya Alaro river confirmed the presence of these substances in Nigerian watercourses including some that have not previously been observed in African rivers more widely. These were from a wide range of therapeutic classes including anti-inflammatories, antidepressants and antihistamines, and were present at relatively high concentrations compared to some other drugs that were measured. Pseudo-persistence was observed, presumably due to continuous discharge of effluents to the river, similar to that found in other studies. One of the key findings of the current study is that pharmaceutical concentrations in the environment are often two to three orders of magnitude higher than typically reported in Europe and the US where most monitoring has been undertaken (Verlicchi et al. 2012; Hughes et al., 2013; Aus der Beek et al., 2016; Madikizela et al., 2017; Burns et al., 2018). Some of the highest pharmaceutical concentrations ever found in rivers globally have been reported in this paper, it not being uncommon to have measured levels in the tens of micrograms per litre range up to 129 µg L-1. This is in-line with findings in Kenya ((K’Oreje et al., 2012, 2016, 2018). Particular compounds of concern in the Nigerian environment appear to be carbamezapine, cimetidine, fexofenadine, metformin, paracetamol and sulfamethoxazole. The presence of a range of substances at such high concentrations may be attributed to a range of factors including over-the-counter sales, differences in health issues, poorer removal efficiencies at wastewater treatment plants, unregulated discharges by pharmaceutical manufacturing companies, and illegal disposal of sewage by vacuum trucks. Other workers have proposed that these sources are likely to be important in Africa (Fekadu et al., 2019) and further study is needed to disentangle inputs from these various sources.

There were few spatial trends observed in pharmaceutical pollution in this study which appears to be ubiquitous although the absence of many compounds in pharmaceutical production effluent suggests that sewage effluent is the main source of pollution. Indeed, the occurrence and concentrations of pharmaceuticals in wastewater treatment plant effluent and surface water were very similar. This highlights that receiving waters have little capacity to dilute effluent but also that further unregulated and unmonitored sources of effluent may be discharging to the river, such as vacuum trucks collecting effluent in urban areas. Other work in India has however proposed that pharmaceutical production facilities are a key source of pharmaceutical pollution developing countries (Balakrishna et al., 2017).

Although season had an impact on pharmaceutical pollution a complex picture exists with some compounds being found at extremely high concentrations in the dry season and, conversely, others being relatively high during the wet season. Previous studies have proposed a range of reasons for variation across the year, including seasonal usage and changes in environmental conditions (e.g. temperature and river flow) (Tewari et al., 2013; Kolpin et al., 2014; Fekadu et al., 2019). It may be that the multiple sources of pharmaceuticals in the catchment results in the complexity of spatial patterns found in the current study, with some continuous effluent discharges being diluted in the wet season but other sources (e.g. urban waste sites) having pollutants mobilised in periods of rainfall.

## 5. Conclusion

This is the most detailed study to date of pharmaceutical pollution in African river catchments and it has highlighted their occurrence at some of the highest concentrations ever found globally. Concentrations in Nigerian rivers appear to be several orders of magnitude higher than those reported for Europe and the US and, in some cases, even higher than the few existing values produced for other developing countries. Sewage effluent appears to be the key source of pollution although it is speculated that further investigation of unregulated sources is needed. Whilst also important, it appears that many compounds are not discharged from drug manufacturing plants. The complexity of temporal patterns across seasons is proposed to be due to a greater range of sources contributing to pharmaceutical loads than in many existing studies, which poses a particular research challenge for understanding and managing pharmaceutical pollution in African rivers. The scenario presented here has a strong likelihood of being replicated in other major African cities as well as megacities in other developing nations globally, where pharmaceuticals are available over the counter and where wastewater discharges to rivers proceed without regulation. A key implication for the global research agenda on pharmaceutical occurrence, fate and effects is that studies should focus more on developing countries where contamination of water is likely to be most significant.

Acknowledgments

The Federal Government of Nigeria through the Tertiary Education Trust Fund (TETFUND) funded a PhD to Olatayo Michael Ogunbanwo. Lagos State Environmental Protection Agency allowed use of their facilities and personnel. The York Centre of Excellence in Mass Spectrometry was created thanks to a major capital investment through Science City York, supported by Yorkshire Forward with Funds from the Northern Way Initiative, and subsequent support from EPSRC (EP/K039660/1:EP/M028127/1).

REFERENCES

Adegun, O. Odunuga O., Appia, Y.(2015).  Environmental change and ecological services in system 1 drainage area of Lagos. Environ and earth sc, 5 (17).

Agunbiade, F.O., Moodley, B., 2014. Pharmaceuticals as emerging organic contaminants in Umgeni River water system, KwaZulu-Natal, South Africa. Environ. Monit.

Assess. 44, 7273-7291.

Agunbiade,F.O., Moodley, B. (2016). Occurrence and distribution pattern of acidic pharmaceuticals in surface water, wastewater, and sediment of the Msunduzi river, Kwazulu-Natal, South Africa. Environ Tox, Vol.35,No 1, pp. 36-46.

Aherne, G. W., English, J., Marks, V. (1985). The role of immunoassay in the analysis of microcontaminants in water samples. Ecotox Environ Safety 9, 79–83.

Akande, T. M., Ologo, M. O. (2007). Prescription Pattern at a Secondary Health Care Facility in Ilorin, Nigeria. Annals of African Medicine 6 (4): 186 – 189.

Alexy, R., Kümpel, T., Kümmerer, K., (2004). Assessment of degradation of 18 antibiotics in the closed bottle test. Chemosphere 57, 505-512.

Amdany, R., Chimuka, L., Cukrowska, E., (2014). Determination of naproxen, ibuprofen and triclosan in wastewater using the polar organic chemical integrative sampler (POCIS): a laboratory calibration and field application. Water SA 40, 407e414.

Amdany, R., Moya, A., Cukrowska, E., Chimuka, L., (2015). Optimization of the temperature for the extraction of pharmaceuticals from wastewater by a hollow fibre silicone membrane. Anal. Lett. 48, 2343-2356.

Andreozzi, R., Marotta, R., Paxéus, N., 2003. Pharmaceuticals in STP effluents and their solar photodegradation in aquatic environment. Chemosphere.

Ashton, D.; Hilton, M.; Thomas, K. V. (2004).  Investigating the environmental transport of human pharmaceuticals to streams in the United Kingdom. Sci. Total Environ.  333 (1-3), 167-184.

Balakrishna K., Rath A, Praveenkumarreddy Y., Guruge K,  Subedi B.(2017). A review of the occurrence of

pharmaceuticals and personal care products in Indian water. Bodies. Ecotox Environ Safety 137, 113-  120.

Białk-Bielinska, A., Stolte, S., Arning, J., Uebers, U., Beoschen, A., Stepnowski, P., Matzke, M., (2011). Ecotoxicity evaluation of selected sulfonamides. Chemosphere 85, 928e933.

Bound, J.P., Voulvoulis, N. (2006). Predicted and measured concentrations for selected pharmaceuticals in UK rivers: implications for risk assessment. Water Res. 40 (15), 2885-2892.

Boxall, A.B.A, Blackwell P., Cavallo, R., Kay, P., Tolls, J. (2002). The sorption and transport of a sulphonamide antibiotic in soil systems.  Toxicol Lett. 131:19-28.

Bringolf, R. B.; Heltsley, R. M.; Newton, T. J.; Eads, C. B.; Fraley, S. J.; Shea, D.; Cope, W. G. (2010). Environmental occurrence and reproductive effects of the pharmaceutical fluoxetine in native freshwater mussels. Environ. Toxicol. Chem.  29 (6), 1311-1318.

Bruchet A, Hochereau C, Picard C, Decottignies V, Rodrigues JM, Janex-Habibi ML. (2005). Analysis of drugs and personal care products in French source and drinking waters: The analytical challenge and examples of application. Water Sci Technol 52:53–61.

Burns, E.E., Thomas-Oates, J., Kolpin, D.W., Furlong, E.T. and Boxall, A., 2017. Are exposure predictions, used for the prioritization of pharmaceuticals in the environment, fit for purpose?. Environmental Toxicology and Chemistry. 36(10), p. 2823-2832.

Caldwell, D. J., Mastrocco, F., Hutchinson, T. H., Lange, R., Heijerick, D., Janssen, C, Anderson, P. D., Sumpter J. P. (2008).  Derivation of an aquatic predicted no-effect concentration for the synthetic hormone, 17α-ethinyl estradiol. Environ Sci Technol 42, 7046–7054

Camacho-Munoz, D.; Martin, J.; Santos, J. L.; Aparicio, I.; Alonso, E. (2010). Occurrence, temporal evolution and risk assessment of pharmaceutically active compounds in Donana Park (Spain) main watersheds.  J. Hazards.  Mater., 183 (1-3), 602-608.

Camacho-Munoz, M. D.; Santos, J. L.; Aparicio, I.; Alonso, E. (2010). Presence of pharmaceutically active compounds in Donana Park (Spain) main watersheds.  J. Hazards.  Mater. 2010, 177 (1-3), 1158-1162..

Castiglioni, S., Pomati, F., Miller, K., Burns, B.P., Zuccato, E., Calamari, D., Neilan, B.A., (2008). Novel homologs of the multiple resistance regulator maras in antibiotic contaminated environments. Water Res. 42, 4271-4280.

Cleuvers, M., (2003). Aquatic ecotoxicity of pharmaceuticals including the assessment of combination effects. Toxicol. Lett. 142, 185e194.

Commission Implementing Decision (EU) 2015/495 of 20 March 2015 establishing a watch list of substances for Union-wide monitoring in the field of water policy pursuant to Directive 2008/105/EC of the European Parliament and of the Council.

Crane, M.; Watts, C.; Boucard, T.(2006). Chronic aquatic environ-mental risks from exposure to human pharmaceuticals. Sci. Total Environ. 367 (1), 23-41.

Daughton CG (2004). PPCPs in the environment: future research—beginning with the end always in mind. In:

Kümmerer K, ed. Pharmaceuticals in the environment, 2nd ed. Springer, pp. 463–495.

Deneshvar, A.; Svanfelt, J.; Kronberg, L.; Prevost, M.; Weyhenmeyer, G. A. (2010).  Seasonal variations in the occurrence and fate of basic an neutral pharmaceuticals in a Swedish river-lake system.  Chemosphere, 80 (3), 301-309.

Ebele,A.J, Abdallah, M.A,, Harrad, S, (2017). Pharmaceuticals and personal care products (PPCPs) in freshwater aquatic environment. Emerging contaminants 3, 1-16.

Fatta-Kassinos, D., Meric, S., Nikolaou, A., (2011). Pharmaceutical residues in environmental waters and wastewater. Anal. Bioanal. Chem. 399, 251-275.

Fekadu, S., Alemayehu, E., Dewil, R. and Van der Bruggen, B., 2018. Pharmaceuticals in freshwater aquatic environments: a comparison of the African and European challenge. Science of the Total Environment.

Fent, K; Weston, A. A.; Caminada, D. (2006). Ecotoxicology of human pharmaceuticals.  Aquat. Toxicol. 76 (2), 122-159.

Fernandez, C.; Gonzalez-Doncel, M.; Pro, J.; Carbonell, Tarazona, J. V. (2010). Occurrence of pharmaceutically active compounds in surface waters of the Henares-Jarama-Tajo river system (Madrid, Spain) and a potential risk characterisation.  Sci. Total Environ. 408 (3), 543-551.

Fick, J., Lindberg, R.H., Parkkonen, J., Arvidsson, B., Tysklind, M., Larsson, D.G.L. Therapeutic levels of

levonorgestrel detected in blood plasma of fish: results from screening rainbow trout exposed to treated

sewage effluents, Environ. Sci. Technol. 44 (2010) 2661-2666.

Furlong, E.T., Noriega, M.C., Kanagy, C.J., Kanagy, L.K., Coffey, L.J. and Burkhardt, M.R., 2014. Determination of human-use pharmaceuticals in filtered water by direct aqueous injection: high-performance liquid chromatography/tandem mass spectrometry (No. 5-B10). US Geological Survey.

Garcia-Galan, M.; Villagrasa, M.; Silvia Diaz-Cruz, M.; Barcelo, D., (2010). LC-QqLIT MS analysis of nine sulfonamides and one of their acetylated metabolites in the Llobregat River basin. Quantitative determination and qualitative evaluation by IDA experiments. Anal. Bioanal. Chem.  397, (3).

Gracia-Lor, E., Sancho, J.V., & Hernandez, F. (2010).  Simultaneous determination of acidic, neutral and basic pharmaceuticals in urban wastewater by ultra high-pressure liquid chromatograpgy-tandem mass spectrometry. Journal of Chromatorgraphy A, 1217, 622-632..

Hughes, S.R., Kay, P., Brown, L.E. (2013). Global synthesis and critical evaluation of pharmaceutical data sets collected from river systems. Environ. Sci. Technol., 47, (2) 661-677.

Jerker F;  Tomas B;, Martina H; Jonatan K;  Micael J.(2017). Screening of benzodiazepines in thirty European

rivers. Chemosphere 176:324-332.

Kasprzy-Hordem, B.; Dinsdale, R. M; Guwy, A. J. (2009). The removal of pharmaceuticals, personal care products, endoctrine disruptors and illicit drugs during waste water treatment and its impact on the quality of receiving waters. Water Res. 2009, 43 (2), 363-380.

Kasprzyk-Horden, B., Dinsdale, R. M. Guwy, A. J., (2017).  Multi-residue method for the determination of basic/neutral pharmaceuticals and illicit drugs in surface water by solid-phase extraction and ultra performance liquid chromatography-positive electrospray ionisation tandem mass spectrometry. J. Chromatogr. A 1161 (1-2), 132-145.

Kay, P., Hughes, S.R., Ault, J. R., Ashcroft, A. E., Brown, L. E. (2017).  Widespread, routine occurrence of pharmaceuticals in sewage effluent, combined sewer overflows and receiving waters. J. Envpol. 10 (087),

0269-7491.

K’Oreje, K.O., Vergeynst L., Ombaka D., De Wispelaere P., Okoth M.,Langenhove H., Demeestere K.,  (2016).

Occurrence patterns of pharmaceutical residues in wastewater, surface water, and groundwater of Nairobi

and Kisumu city,    Kenya. Chemosphere 149, 238-244

Kim, S. C.; Carlson, K. (2007).  Temporal and spatial trends in the occurrence of human and veterinary antibiotics in aqueous and river sediment matrices. Environ. Sci. Technol.  41, 50-57.

Kolpin, D.W., Skopec, M., Meyer, M.T., Furlong, E.T., Zaugg, S.D., (2014). Urban contribution of pharmaceuticals and other organic wastewater contaminants to streams during differing flow conditions. Sci. Total Environ. 328, 119–130

Kostish, M. S.; Batt, A. L.; Glassmeyer, S. T.; Lazorchak, J. M. (2010).  Predicting variability of aquatic concentrations of human pharmaceuticals.  Sci. Total Environ. 408 (20), 4504-4510.

Lacey, C., McMahon, G., Bones, J., Barron, L., Morrissey, A., & Tobin, J. (2008).  An LCMS method for the determination of pharmaceutical compounds in wastewater treatment plant influent effluent samples.  Talanta, 75, 1089-1097.

Larsson, E., al-Hamimi, S., Jonsson, J.A., (2014). Behaviour of nonsteroidal anti-inflammatory drugs and eight of their metabolites during wastewater treatment studied by hollow fibre liquid phase microextraction and liquid chromatography-mass spectrometry. Sci. Total Environ. 485e486, 300-308.

Lei Jiang; Xialin hub; Daqiang Yin; Hongchang Zhang; Zhenyang Yu (2011). Occurrence, distribution  and seasonal variation of antibiotics in the River, Shanghai, China.

Chemosphere 82, 822–828.

Li, H. X.; Helm, P.A.; Metcalfe, C. D. (2010). Sampling in the Great Lakes for pharmaceuticals, personal care products and endocrine disrupting substances using the passive polar organic chemical integrative sampler.  Environ. Toxicol. Chem.  29(4), 751-762..

Lopez-Roldan, R; de Alda, M.L.; Gross M.; Petrovic, M; Martin-Alonso, J.; Barcelo, D. (2010). Advanced monitoring of pharmaceuticals and estroens in the Liobregat River basin (Spain) by liquid chromatophraphy-triple quadrupole-tandem mass spectrometry in combination with yultra performance liquid chromatrophy-time of flight-mass spectrometry.  Chemosphere. 80 (11), 1337-1344.

Loraine, G. A.; Pettigrove, M. E. (2006). Seasonal variations in concentrations of pharmaceuticals and personal care products in drinking water and reclaimed wastewater in Southern California. Environ. Sci. Technol. 40 (3), 687-695. (139)

Madikizela, L.M., Tavengwaa, N.T., and Chimuka, L., (2017). Status of pharmaceuticals in African water bodies: Occurrence, removal and analytical methods.Environ Management 193 (2017) 211-220.

Matongo, S., Birungi, G., Moodley, B., Ndungu, P., 2015a. Pharmaceutical residues in water and sediment of

msunduzi river, KwaZulu-Natal, South Africa. Chemosphere 134, 133-140.

Matongo, S., Birungi, G., Moodley, B., Ndungu, P., 2015b. Occurrence of selected pharmaceuticals in water and

sediment of Umgeni River, KwaZulu-Natal, South Africa. Environ. Sci. Pollut. Res. 22, 10298e10308.

Ngumba E., Gachanja A., Tuhkanen T., (2016a). The occurrence of selected antibiotics and antiretroviral drugs

in Nairobi River Basin, Kenya. Science of the Total Environment, 539, 206-213

Ngumba, E., Kosunen, P., Gachanja, A., Tuhkanen, T., 2016b. A multiresidue analytical  method for trace level

determination of antibiotics and antiretroviral drugs in wastewater and surface water using SPE-LC-

MS/MS and matrix-matched standards. Anal. Methods 8, 6720-6729.

https://doi.org/10.1039/C6AY01695B.

Nicholas J. Niemuth and Rebecca D. Klaner, (2015). Emerging wastewater contaminant metformin causes

intersex and reduced fecundity in fish.Chemosphere, vol 135, 38-45.

Norpoth K, Nehrkorn A, Kirchner M, Holsen H, Teipel H (1973) Investigations on the problem of solubility and stability of steroid ovulation inhibitors in water,     wastewater and activated sludge. Zbl Hyg I Abt Orig B 156: 500–511

Nwolisa C.E., Erinaugha E.U., Ofoleta S.I. (2006). Prescribing practices of doctors attending to under-fives in a children’s outpatient clinic in Owerri, Nigeria.  J. Tropical Pediatrics. 52(3): 197-200.

Odusanya O. O. (2005). Drug use indicators at a secondary health care facility in Lagos, Nigeria. J. Community Medicine Primary Health Care. 16(1):21-24

Ogunbanwo,O.M.A.(2011). Distribution and variation of macrobenthic invertebrates in Ikorodu axis of Lagos Lagoon. Unpublished manuscript, M.Sc thesis, University of Lagos, Lagos State, Southjwest Nigeria.

Oilers, S.; Singer, H. P.; Fassler, P.; Muller, S. R. (2001). Simultaneous quantification of neutral and acidic pharmaceuticals and pesticides at the low ng/L level in surface and wastewater. J. Chromatogr. 911 (2), 225-234.

Onesios, K.M., Yu, J.T., Bouwer, E.J., (2009). Biodegradation and removal of pharmaceuticals and personal care products in treatment systems: a review. Biodegradation 20, 441–466.

Ort, C. Madikizela, L.M., Rieckermann, J.R., Joss, A., (2010).  Sampling for pharmaceuticals and personal care products (PPCPs) and illicit drugs in wastewater systems: are your conclusions Valid? A critical review.  Environ. Sci. Technol. 44 (16), 6024-6035.

Oshikoya K. A., Ojo  O. L. (2007), Medication errors in paediatric outpatient prescriptions of a  teaching hospital in Nigeria. Nig Q. J.  Hosp. Med. Apr-Jun. 17(2):74-8

Peng, X., Wang, C., Zhang, K., Wang, Z., Huang, Q., Yu, Y., Ou, W., 2014. Profile and behavior of antiviral

drugs in aquatic environments of the Pearl River Delta,China. Sci. Total Environ. 466-467, 755-761.

https://doi.org/10.1016/J.SCITOTENV.2013.07.062

Santos, L.H.M.L.M., Araujo, A.N., Fachini, A., Delerue-Matos, C., Montenegro, M.C.B.S.M., (2010). Ecotoxicological aspects related to the presence of pharmaceuticals in the aquatic environment. J. Hazard. Mater. 175 (1-3).

Stumpf, M.; Ternes, T. A.; Wilken, R. D.; Rodrigues, S. V.; Baumann, W. (2009).  Polar drug residues in sewage and natural waters in the state of Rio de Janeiro, Brazil. Sci. Total Environ.  225 (1-2), 135-141.

Tabak, H.H., Bunch R.L. (1970) Steroid hormones as water pollutants. In: Developments in Industrial Microbiology. Washington, 367–376

Tauxe-Wuersch A, De Alencastro LF, Grandjean D,  Tarradellas J (2005).  Occurrence in sewage treatment plants and risk assessment of several acidic drugs in Switzerland.  Water Res. 39:1761-1772.

Ternes, T. A. (1998).  The occurrence of drugs in German sewage treatment plants and rivers.  Waters Res. 32 (11), 3245-3260.

Tewari, S., Jindal, R., Kho, Y.L., Eo, S., Choi, K., (2013). Major pharmaceutical residues in wastewater treatment plants and receiving waters in Bangkok, Thailand, and associated ecological risks. Chemosphere 91, 697–704.

Tim Aus Der Beek, Frank-Andreas Weber, Axel Bergmann, Silke Hickmann, Ina Ebert, Arne Hain and Anette Kuster., 2016. Pharmaceuticals in the environment-Global occurrence and perspectives. Environ Toxi and Chem. 35 (4) 823-835.

Tixier, C.; Singer, H. P.; Oellers, S.; Muller, S. R. (2003). Occurrence and fate of carbamazepine, clofibric acid, diclofenac, ibuprofen, ketoprofen, and naproxen in surface waters. Environ. Sci. Technol. 37 (6), 1061-1068.

Togola, A.; Budzinki, H. (2007). Analytical development for analysis of chem. 388 (3), 627-635.

Triebskorn R, Schwarz S, K€ohler H-R, Berg K, Jungmann D, Frey M, Oehlmann J, Oetken M. (2015). From theory to reality—Evaluation of suitable organisms and test systems for the biomonitoring of pharmaceuticals. Part I: Literature review. UBA-Texte 64/2014.

U.S. Geological Survey, 2011, Application of the result-level ‘v’ value qualifier code and ‘E’ remark code to selected organic results reported by the National Water Quality

Laboratory (NWQL): U.S. Geological Survey Office of Water Quality Technical Memorandum 2012.01, accessed May 13, 2013, at

http://water.usgs.gov/admin/memo/QW/qw12.01.pdf.

Van Nuijs, A. L. N.; Pecceu, B.; Theunis, L.; Dubois, N.; Charlier, C.; Jorens, P. G.; Bervoets, L.; Blust, R.; Neels, H.; Covaci, A. (2009). Spatial and temporal variations in the occurrence of cocaine and benzoylecgonine in waste- and surface water from Belgium and removal during wastewater treatment. Water Res. 43 (5), 1341-1349.

Vanderford, B. J.; Snyder, S. A. (2006). Analysis of pharmaceuticals in water by isotope dilution liquid chromatography/tandem mass spectrometry. Environ. Sci. Technol. 40 (23), 7312-7320.

Vieno, H., Harkki, H., Tuhkanen, T., Kronberg, L. Occurrence of pharmaceuticals in river water and their elimination in a pilot-scale drinking water treatment

plant, Environ. Sci. Technol. 41 (2007) 5077e5084

Waiser, M. J. Humphries, D.; Tumber, V.; Holm, J. (2011).  Effluent-dominated streams.  Part 2: Presence and possible effects of pharmaceuticals and personal care products in Wacana Creek, Saskatchewan, Canada Environ.  Toxicol. Chem. 30 (2), 508-519.

Watkinson, A. J.; Murby, E. J.; Kolpin, D.W.; Costanzo, S. D. (2009).  The occurrence of antibiotics in an urban watershed:  From wastewater to drinking water.  Sci.  Total Environ.  407 (8), 2711-2723.

Weigel, S.; Berger, U.; Jensen, E.; Kallenborn, R.; Thoresen, H.; Huhnerfuss, H. (2004). Determination of selected pharmaceuticals and caffeine in sewage and seawater from Tromso/Norway with emphasis on ibuprofen and its metabolites. Chemosphere 56 (6), 583-592.

Wilde, F.D., Radtke, D.B., Gibs, Jacob, and Iwatsubo, R.T., 2004 with updates through 2009, Processing of

water samples (version 2.2): U.S. Geological Survey Techniques of Water-Resources Investigations,

book 9, chap. A5, accessed July 2, 2012, at

http://water.usgs.gov/owq/FieldManual/chapter5/html/Ch5_contents.html.

Wishart DS, Feunang YD, Guo AC, Lo EJ, Marcu A, Grant JR, Sajed T, Johnson D, Li C, Sayeeda Z,

Assempour N, Iynkkaran I, Liu Y, Maciejewski A, Gale N, Wilson A, Chin L, Cummings R, Le D, Pon

A, Knox C, Wilson M. DrugBank 5.0: a major update to the DrugBank database for 2018. Nucleic

Acids Res. 2017 Nov 8. doi: 10.1093/nar/gkx1037. PubMed: 29126136

Wood, T.P., Basson, A.E., Duvenage, C., Rohwer, E.R., 2016. The chlorination behaviour and environmental

fate of the antiretroviral drug nevirapine in South African surfacewater.  Water Res. 104, 349-360.

https://doi.org/10.1016/ J.WATRES.2016.08.038.

World Health Organization. (2012). Pharmaceuticals in drinking water. Geneva, Switzerland.

Xu, W. H.; Zhang, G.; Zou, S. C.; Li, X. D.; Liu, Y. C. (2007).  Determination of selected antibiotics in the Victoria Harbour and the Pearl River, South China using high-performance liquid chromatography-electrospray ionisation tandem mass spectrometry. Environ. Pollut. 145 (3), 672-679.

Yamamoto, H., Nakamura, Y. Morigushi, S., Nakamura, Y., Honda., Y., Tamura, I., Hirata, Y., Hayashi, A., Sekizawa, J., (2009). Persistence and partitioning of eight selected pharmaceuticals in the aquatic environment: laboratory photolysis, biodegradation, and sorption experiments.  Water Res. 43 (2). 351-362.

Zhang, S. Y.; Zhang, Q A.; Darisaw, S.; Ehie, O.; Wang, G. D. (2007). Simultaneous quantification of polycyclic aromatic hydrocarbons (PAHs), polychlorinated biphenyls (PCBs), and pharmaceuticals and personal care products (PPCPs) in Mississippi river water, in New Orleans, Louisiana, USA. Chemosphere 66 (6), 1057-1069.

Zhang, Y. J.; Geissen, S. U.; Gal, C. (2008). Carbamazepine and diclofenac: Removal in wastewater treatment plants and occurrence in water bodies. Chemosphere 73 (8), 1151-1161.

Zhou, J. L.; Zhang, Z. L.; Banks, E.; Grover, D.; Jiang, J. Q (2009). Pharmaceutical residues in wastewater treatment work effluents and their impact on receiving river water. J. Hazard. Mater. 166 (2-3), 655-661.

Zuccato, E.; Castigglioni, S.; Bagnati, R.; Melis, M.; Fanelli, R. (2010). Source, occurrence and fate of antibiotics in the Italian aquatic environment.  J. Hazard. Mater.  179 (1-3), 1042 – 1048.